GSM Requirements
for unpowered flight

Made by:
- Michał Skiba

Approved by:
- Maciej Grzywka

Table of Contents:

1.	Introduction	3

1.1.	LTE module	3

1.2.	Micro python	3

1.3.	Raspberry pico	3

1.3.1.	Pins definition	3

2.	Main functionalities	3

2.1.	Detecting if model has LTE coverage	3

2.2.	Connecting to ground base server	3

2.3.	Sending data in packages	3

3.	Required class methods	4

3.1.	.checkCoverage() -> bool	4

3.2.	.checkConnectionWithServer() -> bool	4

3.3.	.sendData(data: bytearray) -> bool	4

4.	Required endpoints	4

4.1.	GET /testConnection	4

4.2.	POST /sendData	4

# Introduction

There must be created Class for communication between rocket and ground base via LTE

## LTE module

Modele used for communication is NB-IoT GSM SIM7020E - Waveshare 19622

## Micro python

Main programing language is micro python, and so created class must be compatible with it.

## Raspberry pico

Code must be compatible with Raspberry pico.
LTE module will be communicating with Raspberry pico via UART.

### Pins definition

- TX – GP0

- RX – GP1

- PWR – GP14

- DTR – GP17

# Main functionalities

## Detecting if model has LTE coverage

There is assumed that module will not have coverage inside diffuser (assumption must be validated). To detect if parachute and inlet cone were ejected main code will periodically check if module has LTE coverage (assuming that after inlet cone is ejected module will regain LTE coverage).

## Connecting to ground base server

After the module regains LTE coverage it must connect/confirm connection with ground base server.

## Sending data in packages

After the module connect/confirm connection with ground base server it must be able to send binary data to server via REST API.

# Required class methods

Class methods correspond to chapters in 2.Main functionalities

## .checkCoverage() -> bool

Returns True if there is LTE coverage
Return False if there is no coverage

## .checkConnectionWithServer() -> bool

Return True if there is a successful connection with server
Return False if there is no connection with server

## .sendData(data: bytearray) -> bool

Return True if data is sent successfully 
Return False if data failed to be sent

# Required endpoints

## GET /testConnection

## POST /sendData