# Injector test
test of orifice injector using 3d printer nozzles

Made by:
- Michał Skiba

Approved by:
-

Table of Contents:

1.	HUB’s	3

2.	List of sensors	3

2.1.	Pressure sensors	3

2.2.	Temperature sensors	4

2.3.	Miscellaneous sensors	5

3.	Sensor’s schematic	5

3.1.	Pressure sensors	5

3.2.	Temperature sensors	6

3.3.	Miscellaneous sensors	6

4.	Phases of flight	7

4.1.	Boost phase	8

4.2.	Separation	8

4.3.	Unpowered ascent	8

4.4.	Drogue parachute ejection	9

4.5.	Descent on Drogue	9

4.6.	Main parachute unfold	9

4.7.	Descent on Main parachute	10

4.8.	Auxiliary buoyancy system (ABS) opening	10

4.9.	Recovery	10

# Introduction

The goal of this test Is to measure how different hole size injectors atomize the kerosene. Additionally, it can be measured volumetric and mechanical efficiencies of pump.

# Setup

Figure 2.1 Experiment setup

| 22 | 2 | Hydraulic nipple 12x1.5 6L - 16x1.5 10L |
| --- | --- | --- |
| 21 | 1 | Fuel tank |
| 20 | 1 | Nut M12x1.5 |
| 19 | 2 | Hydraulic nipple BSP 3/8'' - M12x1.5 |
| 18 | 2 | Coupling (mufa) BSP 1/2'' - M16x1.5 |
| 17 | 2 | Hydraulic nipple M12x1.5 - M12x1.5 |
| 16 | 1 | Power supply for pump motor |
| 15 | 1 | ECS |
| 14 | 1 | Electric motor |
| 13 | 1 | Gear transmission |
| 12 | 1 | Gear pump |
| 11 | 1 | Flow meter |
| 10 | 1 | Manifold block |
| 9 | 1 | Tested injectors |
| 8 | 1 | Background bord for good contrast |
| 7 | 1 | Slow motion camera |
| 6 | 1 | Pressure transducer |
| 5 | 1 | Resistor (55-160 Ohm) |
| 4 | 1 | Power supply for pressure sensor and flow meter (9-18V) |
| 3 | 1 | Micro USB - USB cable |
| 2 | 1 | Raspberry pi Pico |
| 1 | 1 | Laptop |
| No | Qty | Name |